UTCI-DRIVEN CELLULAR AUTOMATA

Generative Thermal Shading and Green Space

for Bangkok 2070 Outdoor Overheating

Atin Punyanitya (65020505) KMITL — Faculty of Architecture ¦ Year 5 Thesis ¦ June 2026 Internship: IRIs A49 ¦ Tools: Grasshopper · Ladybug · CURA-lab · Anemone

# 1.  Problem Statement

Bangkok is projected to become ASEAN's hottest major city by 2050. Under the SSP5-8.5 scenario at 2070, a UTCI analysis of a generic Bangkok block reveals a critical floor shift: the entire distribution of outdoor thermal stress has moved up one category.

| Metric | Present EPW | 2070 SSP5-8.5 | Shift |
| --- | --- | --- | --- |
| UTCI Minimum | 33.8°C (Strong) | 38.2°C (Very Strong) | +4.4°C |
| UTCI Average | 37.6°C (Strong) | 42.2°C (Very Strong) | +4.6°C |
| UTCI Maximum | 42.3°C (V. Strong) | 47.6°C (Extreme) | +5.3°C |
| Hours ≥35°C / yr | 149 hrs | 1,261 hrs | ×8.5 |
| Hours ≥38°C / yr | 3 hrs | 186 hrs | ×62 |

KEY FINDING The future minimum (38.2°C) exceeds the current average (37.6°C). No outdoor grid point or time period falls below ‘Very Strong’ stress in 2070. Conventional passive design — shade, orientation, greenery — is necessary but insufficient. It cannot return outdoor space to comfort. It only reduces how deep into danger you are.

This creates a spatial problem that cannot be solved by intuition. The form of any outdoor intervention must be derived from the data: which cells are hottest, by how much, for how many hours, and what spatial configuration of shade and vegetation is required to bring the maximum number of them below a survivable threshold.

# 2.  Threshold Selection

Three thresholds are relevant to this workflow, operating at different levels of argument:

| Threshold | Value | Meaning | Role in Workflow |
| --- | --- | --- | --- |
| T₁ — Survivability | Wet-bulb 35°C ≈ UTCI 46–48°C | Physiological upper limit. Above this, thermoregulation fails within hours (Sherwood & Huber 2020; PNAS 2023). | Upper bound argument. Maps to 2070 peak exposure (47.6°C). |
| T₂ — T_target | UTCI 41°C | Just below the 2070 projected average (42.2°C). Targets conditions worse than the mean. Grounded in your own UTCI data. | Active CA threshold. Seeds the hot cell set H₀. |
| T₃ — Floor | UTCI 38°C | 2070 future minimum — no cell falls below this. The entire distribution is above very strong stress. | Baseline comparator. Used for iteration convergence check. |

DESIGN IMPLICATION T_target = 41°C is recommended as the active simulation threshold. This is defensible as: ‘we target outdoor conditions exceeding the 2070 projected mean UTCI’ — grounded in your own experimental data, not a generic standard.

# 3.  System Overview

The workflow combines three components into a single feedback loop: UTCI thermal simulation, cellular automata spatial logic, and re-simulation. The loop runs until a convergence condition is met.

| Environmental Driver | CA + Feedback Loop |
| --- | --- |
| Morphed EPW 2070 (SSP5-8.5) Source: CURA-lab Future Weather tool Fields used: dry bulb, dew point, relative humidity, GHR, DNR, wind direction, wind speed. This defines the thermal envelope the building must perform against. | Anemone loop in Grasshopper Each iteration: UTCI sim → hot cell filter → CA neighbourhood expansion → classify → extrude → re-sim. Terminates when coverage % of H₀ is met or maximum generations reached. |

PRECEDENT This workflow is structurally equivalent to URBiiLAB's SDCA (Luitjohan, Ashayeri, Abbasabadi — SimAUD 2022), which uses solar radiation as the CA driver for high-rise massing. The key distinction: SDCA optimises envelope energy harvesting for the building. This workflow optimises thermal survivability for the people in public space.

# 4.  Step-by-Step Process

| 01 | Build the Cell Grid |
| --- | --- |

Divide the site surface into a 1×1 m grid using Grasshopper's Divide Surface component. U count = num_cols, V count = num_rows. This produces two parallel flat lists in row-major order: mesh faces and face-centre points.

Immediately wire these into a Python component that builds the master cell record and a lookup dictionary. This happens before any simulation runs.

### Python Component — Grid Builder

# Inputs: pts (Point3d list), meshes (Mesh list), num_cols (int), num_rows (int)

num_cols = int(num_cols)

num_rows = int(num_rows)

cells = []

cell_lookup = {}

for i, (pt, mesh) in enumerate(zip(pts, meshes)):

row = i // num_cols

col = i %  num_cols

cell = { "idx": i, "row": row, "col": col,

"pt": pt, "mesh": mesh,

"utci": None, "state": "empty", "gen": 0 }

cells.append(cell)

cell_lookup[(row, col)] = cell

OUTPUTS cells — flat list of dicts, one per cell. cell_lookup — (row,col) → cell dict. pts_out — pass-through for UTCI sensors. meshes_out — pass-through for display.

WARNING Verify UV order before proceeding: colour cells by flat index and confirm the gradient reads left→right across row 0. If it spirals or jumps, reparameterise the surface first. This is the most common setup failure.

| 02 | Run UTCI Simulation |
| --- | --- |

Feed pts_out (face centres) directly into the Ladybug UTCI component as sensor positions. Wire the morphed 2070 EPW as the weather file. The output is a flat list of N floats — one per sensor point, in the same order as the cells list. This aligns 1:1 with the cells list by position.

After UTCI runs, inject the values back into the cell records via a second Python component:

# Inputs: cells (list of dicts), utci_values (list of floats)

for cell, val in zip(cells, utci_values):

cell["utci"] = float(val)

| UTCI Range | Stress Category | 2070 Bangkok Status |
| --- | --- | --- |
| ≥ 46°C | Extreme heat stress | Peak exposure (47.6°C max) |
| 38–46°C | Very strong heat stress | Entire 2070 distribution falls here |
| 32–38°C | Strong heat stress | Present average zone (37.6°C avg) |
| 26–32°C | Moderate heat stress | Not reachable in 2070 |

| 03 | Identify Hot Cells H₀ |
| --- | --- |

Scan the cells list and collect every cell where utci > T_target. This set is called H₀ — the seed population for the CA. These are the cells that must be addressed by shade or vegetation.

T_target = 41.0   # input parameter — UTCI degrees Celsius

H0 = []

for cell in cells:

if cell["utci"] is not None and cell["utci"] > T_target:

cell["state"] = "hot"

H0.append(cell)

H₀ is the spatial record of the 2070 UTCI distribution on your specific site. Its shape, density, and location are entirely determined by the simulation — not by designer intuition. This is the key departure from conventional passive design.

| 04 | CA Neighbourhood Expansion |
| --- | --- |

For every cell in H₀, find all neighbouring cells that are not yet assigned a state. These candidate cells N₀ form the frontier of the intervention zone. Two neighbourhood modes are available:

| Moore neighbourhood (8-cell) | Von Neumann neighbourhood (4-cell) |
| --- | --- |
| All 8 surrounding cells including diagonals. Produces diagonal growth, smoother more continuous forms. Recommended for spatial quality and closer to how thermal gradients actually spread. | Only the 4 orthogonal neighbours. Strictly rectilinear, fragmented, more structural. Closer to a column grid logic. Useful if the structural system is orthogonal. |

### Python — Neighbour Lookup Function

def get_neighbours(cell, cell_lookup, num_rows, num_cols, mode="moore"):

r, c = cell["row"], cell["col"]

if mode == "moore":

offsets = [(dr,dc) for dr in [-1,0,1] for dc in [-1,0,1]

if (dr,dc) != (0,0)]

else:  # von neumann

offsets = [(-1,0),(1,0),(0,-1),(0,1)]

result = []

for dr, dc in offsets:

nr, nc = r+dr, c+dc

if 0 <= nr < num_rows and 0 <= nc < num_cols:

nb = cell_lookup.get((nr, nc))

if nb and nb["state"] == "empty":

result.append(nb)

return result

| 05 | Classify: Shade Box or Planting Cell |
| --- | --- |

Each candidate cell in N₀ is evaluated by the classifier. It becomes either a shade box, a planting cell, or is skipped. The classification uses three inputs: sky view factor (SVF from Ladybug), a soil feasibility flag, and a green_ratio parameter.

| Output State | Condition | Architectural Meaning |
| --- | --- | --- |
| shade | SVF > 0.7, or fallback | Extruded box geometry. Blocks direct solar radiation. Height is data-driven. |
| green | SVF 0.3–0.7, soil flag true, rand() < green_ratio | Planted cell. Provides canopy shade, lowers MRT via albedo, cools air via evapotranspiration. |
| skip | Already assigned in a prior generation | Frozen. CA rule is monotonic — states only accumulate, never revert. |

# Inputs: candidates (list), green_ratio (float 0-1), current_gen (int)

import random

for cell in candidates:

if cell["state"] != "empty": continue

svf  = cell.get("svf",  0.8)

soil = cell.get("soil", True)

if soil and svf < 0.7 and random.random() < green_ratio:

cell["state"] = "green"

else:

cell["state"] = "shade"

cell["gen"] = current_gen

green_ratio PARAMETER A single slider controlling the proportion of candidates assigned as planting cells. 0.3 = 30% green, 70% shade. This is the design parameter that determines the spatial character of the output without changing the thermal logic. Vary it to produce a range of geometry options.

| 06 | Extrude Geometry from State |
| --- | --- |

Each classified cell produces a different physical object based on its state and the generation number it was assigned in.

| Shade Box | Planting Cell |
| --- | --- |
| The mesh face is extruded vertically. Height is a function of solar altitude angle and horizontal distance to the nearest H₀ cell — tall enough to cast shadow onto that hot cell at the critical solar hour. h = dist × tan(90° − solar_altitude) Cells further from H₀ need to be taller → naturally terraced profile from core to edge. | The mesh face is tagged as a planting zone. Visualised as a flat plane or canopy proxy sphere. Its cooling feeds back into UTCI as modified MRT and air temperature within canopy radius. Cooling mechanisms: canopy shade, reduced albedo, evapotranspiration (ET). ET lowers air temperature — not available from shade boxes. |

GENERATION → HEIGHT TAPER The gen field stored on each cell drives height tapering. Generation 1 cells (directly adjacent to H₀) have the shortest required extrusion. Generation 2 cells are further from the hot zone and need to be taller to cast shadow at the same solar angle. This produces a natural taper from structural core to planted periphery — not designed, derived from the data.

| 07 | Feedback Loop — Re-run UTCI |
| --- | --- |

After extrusion, the shade boxes and canopy meshes are fed back into the UTCI component as context geometry. Ladybug re-runs with the new obstruction mesh: boxes cast real shadows, changing the MRT of cells below them. Planted cells contribute an air temperature and MRT reduction within their canopy radius.

The updated UTCI values overwrite cell["utci"] for every cell. H₀ is recalculated from the new values. Cells that are now below T_target have been successfully treated. The remaining hot cells form the new H₀ for the next CA generation.

| Termination Condition | Value | Architectural Meaning |
| --- | --- | --- |
| Coverage % | 80% of original H₀ cells treated | Recommended. The remaining 20% become the intentional thermal gradient — the transition zone outside the building. |
| Max iterations | 5 generations | Hard cap. Prevents runaway growth. Geometry at this point is the output regardless of coverage. |
| H₀ empty | All hot cells below T_target | Full thermal coverage. Rare in practice — likely produces an unbroken canopy with no spatial gradient. |

CRITICAL Do not use H₀ empty as the sole termination condition. A fully converged CA will grow an unbroken canopy that solves the thermal problem but destroys the spatial gradient which is the core architectural argument. Use coverage % termination to preserve the transition zone deliberately.

| 08 | Output Geometry |
| --- | --- |

The algorithm produces a hybrid field of two intervention types, arranged by their data-driven position in the UTCI gradient. Location, height, and density are all consequences of the thermal data.

| Zone | Description | Spatial Character |
| --- | --- | --- |
| Structural core | Dense shade boxes over hottest cells. Tallest extrusions. Gen 1. | Marks the zone of peak 2070 UTCI. The spatial record of the most extreme thermal data. |
| Mixed zone | Shorter boxes and planting cells interleaved. Gen 2. | The civic transition space. Gradient between extreme exterior and conditioned interior. |
| Green edge | Predominantly planted cells. Lowest extrusions. Gen 3+. | Albedo and ET do the work. Softest spatial experience. Entry condition. |
| Untreated cells | Cells never reached by CA. Remain hot. | Not a failure — these are the outside of the building. The datum the architecture departs from. |

| Output | Type | Purpose |
| --- | --- | --- |
| Shade box geometry | Brep / Box list | Baked to Rhino for further design development |
| Canopy meshes | Mesh list | Baked for planting zones and proxy tree placement |
| UTCI delta map | Float list | Per-cell improvement: before vs after, quantified |
| Generation map | Int list / false-colour | Spatial gradient visualisation — thesis diagram |
| Coverage statistic | Float (0–1) | % of original H₀ cells brought below T_target |
| Green / shade ratio | Float | Actual ratio achieved vs green_ratio input |

# 5.  Complete Anemone Loop Structure

The feedback loop is implemented in Grasshopper using the Anemone plugin. The loop component replaces Hoopsnake and is the standard tool for iterative simulation in Grasshopper.

| Iteration | Component Sequence | What Changes |
| --- | --- | --- |
| 0 (init) | Divide Surface → Grid Builder → UTCI → UTCI Inject | Baseline UTCI values populated on all cells. No geometry yet. |
| 1 (gen 1) | Filter H₀ → CA Expand → Classify → Extrude → Context Mesh → UTCI re-sim | First ring of shade boxes and planted cells generated. H₀ updated. |
| 2 (gen 2) | Filter H₀¹ → CA Expand → Classify → Extrude → Context Mesh → UTCI re-sim | Second ring grown from outer edge of gen 1. Geometry accumulates. |
| n (gen n) | Repeat until convergence condition met | Coverage % checked after each iteration. Loop exits when threshold or max gen reached. |

ANEMONE NOTE Anemone requires: Loop Start component (feeds initial data), Loop End component (feeds back updated data), and a Reset input (boolean toggle). The cells list and cell_lookup dict must be passed through Loop End unchanged — they are the persistent state across iterations. Only utci_values, H0, and geometry outputs need to be recalculated each iteration.

# 6.  CA Formal Definition

This workflow uses a seeded growth CA, not Conway’s Game of Life. The distinction matters for understanding the behaviour.

| Conway's Game of Life | This Workflow |
| --- | --- |
| Symmetric, death-capable CA. Cells are born and die based on neighbour counts. The system is unpredictable — patterns emerge, collapse, oscillate. Designed to model population dynamics. | Asymmetric, monotonic CA. No death. States only accumulate. Hot cells are frozen seeds. Growth is directional: from data to periphery. The rule is seeded, not symmetric. |

### Formal notation

A 2D CA is defined as a tuple: (Z², S, N, f)

| Component | Symbol | Definition in this workflow |
| --- | --- | --- |
| Integer lattice | Z² | The 1×1 m grid of (row, col) cell positions on the site surface |
| State space | S | { empty, hot, shade, green } — four possible states |
| Neighbourhood | N | Moore r=1: all (r+Δr, c+Δc) where Δr,Δc ∈ {-1,0,1}, excluding (0,0) |
| Transition rule | f | s(i,j,t+1) = f( s(i,j,t), N(i,j) ) — see rule below |

### Transition rule

s(i,j, t+1) =

"shade"  if s(i,j,t)="empty" AND

∃ neighbour n : s(n,t)="hot" AND rand() > green_ratio

"green"  if s(i,j,t)="empty" AND

∃ neighbour n : s(n,t)="hot" AND rand() ≤ green_ratio

s(i,j,t) otherwise  (hot, shade, green states are frozen)

From generation 2 onwards, the frontier expands: empty cells neighbouring any assigned cell (shade or green) also become candidates, not just cells neighbouring hot cells. This produces the concentric generation banding that encodes the spatial gradient.

# 7.  Comparison to SDCA Precedent

Luitjohan, Ashayeri, Abbasabadi — “SDCA: Context-sensitive Solar-driven High-rise Design Using Generative Cellular Automata” — SimAUD + ANNSIM 2022. This is the closest published precedent.

| Dimension | SDCA (URBiiLAB, 2022) | This Thesis (KMITL, 2026) |
| --- | --- | --- |
| Environmental driver | Solar radiation index (SRI) | UTCI thermal stress (2070 morphed EPW) |
| CA seed condition | High shadow cells (low SRI = blocked) | High UTCI cells (stress > T_target) |
| CA growth direction | Grows toward sun exposure | Grows outward from hot zone perimeter |
| Geometry output | 3D high-rise massing (voxel) | Ground-plane field (shade boxes + planted cells) |
| Feedback loop | Re-sim SRI via Anemone | Re-sim UTCI with context mesh |
| Optimisation goal | Maximise SRI (energy for building) | Minimise UTCI (survivability for people) |
| Scale / type | High-rise, single building | Ground-plane civic public space |
| CA implementation | Ladybug + Anemone + Rabbit | Ladybug + Anemone + Python (custom) |
| Key distinction | Energy harvesting for the building | Thermal survivability for public users |

SHARED LOGIC Both workflows share the same pipeline structure: EPW → simulation → CA seed → CA growth → re-sim feedback loop → form output. SDCA validates the methodological approach at peer-reviewed level. The distinctions above define the original contribution of this thesis.

# 8.  Connection to Thesis Argument

The CA workflow is not form-finding for its own sake. It is the spatial operationalisation of the thesis position: that the 2070 UTCI floor shift requires a new building typology whose spatial logic is entirely derived from climate data.

| Thesis Claim | How the CA Workflow Delivers It |
| --- | --- |
| The future UTCI floor makes all outdoor conditions thermally dangerous | T_target is set at 41°C — below the 2070 mean. H₀ encompasses a large proportion of the site. The scale of the intervention is directly proportional to the floor shift. |
| Passive design is insufficient — cannot return to comfort | The CA cannot bring cells below 26°C (no-stress zone). The termination condition acknowledges this: coverage %, not comfort. The goal is harm reduction, not comfort. |
| Architecture must provide a thermal gradient from exterior to refuge | The generation map is this gradient. Gen 0 = hot exterior datum. Gen 1 = structural shade core. Gen 2–3 = transition. Interior (conditioned) = the building on the other side of the gradient. |
| Form is driven by data, not intuition | The generation map is the proof. It is a false-colour spatial record of which UTCI values, at which locations, produced which geometry. No equivalent diagram is possible for intuitively-designed buildings. |
| Accessible to all classes — civic infrastructure logic | The CA operates on the ground plane of the civic space — the public thermal transit hub. It is not applied to a private building envelope. The spatial distribution it produces is the public space itself. |

—  End of Document  —