THESIS RESEARCH BRIEF

Living Context Document — Upload at the Start of Every Session

| Student | Atin Punyanitya (65020505) |
| --- | --- |
| Institution | KMITL — Faculty of Architecture |
| Stage | Thesis Development ¦ Year 5 |
| Internship | IRIs A49 — current |
| Background | Computational design, environmental analysis tools (Grasshopper, Ladybug/Honeybee, CURA-lab) |
| Last Updated | June 2026 |

# 1. Research Context

Bangkok is projected to become ASEAN's hottest major city by 2050. By 2070 (SSP5-8.5 scenario), a UTCI analysis experiment on a generic Bangkok block shows the following critical finding:

| Critical Finding — UTCI Floor Shift (2070, SSP5-8.5) |  |
| --- | --- |
| Present UTCI min / avg / max | 33.8°C (Strong) / 37.6°C (Strong) / 42.3°C (Very strong) |
| Future UTCI min / avg / max | 38.2°C (Very strong) / 42.2°C (Very strong) / 47.6°C (EXTREME) |
| Key insight | The future minimum (38.2°C) exceeds the current average (37.6°C). No outdoor grid point or time period falls below 'Very strong' stress. The entire distribution has shifted up one category. |
| Hours ≥35°C / yr | Present: 149 hrs → Future: 1,261 hrs (×8.5) |
| Hours ≥38°C / yr | Present: 3 hrs → Future: 186 hrs (×62) |
| Morphing method | Normal Shift Pattern via CURA-lab Future Weather tool (cura-lab.adai.pt), target 2070, SSP5-8.5, all models |
| Design implication | Conventional passive design (shade, orientation, greenery) is necessary but insufficient — it cannot return outdoor space to comfort. It only reduces how deep into danger you are. |

## Urban Heat Inequity — Bangkok

- Bangkok's UHI effect creates district-level temperature disparities — high-risk districts (inner city, low-income, low-green-coverage) are measurably hotter than wealthier tree-covered areas.

- Thermal inequity: lower-income populations have least access to cooling infrastructure while facing highest thermal exposure.

- Vulnerable groups (elderly 65+, children under 15, outdoor laborers) are concentrated in highest-risk districts — mapping this overlap is the site selection method.

- Microburst events (wet microburst / 'rain bomb') are a compounding factor: intense, localized wind downdrafts during storms require structural design responses beyond passive thermal logic.

# 2. Thesis Position

Research Question When Bangkok's outdoor thermal floor shifts above survivability thresholds by 2070, what spatial typology — derived from projected UTCI data, wet-bulb thresholds, and microburst load conditions — can redefine accessible public thermal refuge as civic infrastructure?

## Core Argument

When no outdoor condition in Bangkok is thermally survivable without risk (2070 projection), the spatial relationship between public life and building fundamentally changes. Public activity doesn't disappear — it migrates indoors and into transition zones. The building typology that emerges from this condition is not simply a 'green' or 'passive' building — it is civic thermal infrastructure: a designed gradient between extreme exterior exposure and conditioned interior refuge, accessible to all classes.

What makes this not typical passive design: The form is not driven by shading geometry or orientation intuition. It is driven by the UTCI floor problem — designing spatial sequences, transition zones, and thermal gradients calibrated to the gap between 47.6°C peak exposure and a defined survivable threshold — using data that cannot be felt or guessed by a designer without simulation.

## Proposed Typology — Public Thermal Transit Hub

A new civic building type that sits at the intersection of commuter/pedestrian infrastructure and public social space. Designed so that the daily movement of low-income outdoor workers and vulnerable residents passes through thermally conditioned public space without requiring payment.

This satisfies KMITL's building-type requirement (real program, functional correctness) while being a genuinely new typology — not a wellness centre, not a shopping mall, not a park. A building whose spatial logic is entirely derived from climate data.

Program elements

- Shaded public arrival / transition zone (semi-outdoor, data-driven depth)

- Covered pedestrian corridors connecting to transit / street

- Publicly accessible cooling hall (no entry fee — civic infrastructure logic)

- Community services: water stations, first aid, informal vendor space

- Upper program: community facilities, flexible civic space

- Building envelope / skin: kinetic or data-responsive, calibrated to 2070 UTCI + microburst wind loads

# 3. Methodology — Data to Form Pipeline

| Phase | Data / Method | Architectural Output |
| --- | --- | --- |
| 01 Climate baseline | Morphed EPW (2070, SSP5-8.5) via CURA-lab. Key fields: dry bulb, dew point, relative humidity, GHR, DNR, wind direction, wind speed. | Defines the thermal envelope the building must perform against. Calibrates shading depth, insulation thresholds, wet-bulb survivability hours. |
| 02 UTCI mapping | Grasshopper + Ladybug UTCI analysis on site geometry. Compare present vs 2070 EPW. Identify thermal gradient zones. | Form-finding driver — spatial layout follows the UTCI gradient from extreme exterior to refuge interior. |
| 03 Site selection | Overlay: district UHI risk map × vulnerable population density (elderly + children) × transit accessibility. | Site is not chosen aesthetically — it is the intersection of highest heat risk and highest vulnerable user density. |
| 04 Wind / microburst load | EPW Field 22 peak wind spikes (microburst events). Structural load testing. | Tectonic and structural logic. Kinetic facade safety overrides. Not just shading — storm-resilience as a design generator. |
| 05 Wet-bulb threshold analysis | Relative humidity × dry bulb → wet-bulb temperature. Hours exceeding survivable threshold per year. | Determines how much of the program needs active mechanical cooling vs passive transition vs fully open — and for how many hours per day. |
| 06 Spatial / behavioral data | Pedestrian movement data, occupancy patterns, user group analysis (outdoor laborers, elderly, children). | Program distribution — where each user group moves through the thermal gradient. Space syntax if applicable. |
| 07 Form optimization | Genetic algorithm or multi-objective optimization (MOO) in Grasshopper — simultaneously optimizing UTCI performance, shading coverage, and spatial accessibility. | Final form is the output of competing optimization goals — not a designer's intuition. |

# 4. Theoretical Lineage

From pre-thesis research (A Study on Computational Methods and Workflow to Achieve Data Responsive Design, Atin Punyanitya, 2026):

| Year / Author | Theory | Relevance to Thesis |
| --- | --- | --- |
| 1995 — Frazer | Evolutionary Architecture | Genetic rules + environmental criteria → form generation. Basis for GA/MOO form-finding in Phase 07. |
| 1999 — Lynn | Animate Form | Time and motion as design inputs. Supports kinetic facade logic driven by UTCI + wind data. |
| 2003 — Digital Tectonics | Physical-virtual intersection | Fabrication logic follows computational form — not post-rationalization. |
| 2008 — Schumacher | Parametricism | All elements parametrically linked — change one climate variable, entire system adapts. |

Key computational methods from pre-thesis

- Genetic Algorithm (GA) — structural optimization (Hyundai Motorstudio precedent)

- Multi-Objective Optimization (MOO) — competing goals simultaneously (SERENE project precedent)

- Behavioral Model / Agent-Based — pedestrian movement simulation (Spyropoulos precedent)

- Self-Organizing Maps (SOM) — spatial data clustering and floor plan logic

- Cellular Automata — generative massing rules

# 5. Advisor Feedback & Responses

| Advisor Comment | Current Response / Design Decision |
| --- | --- |
| Search for a new typology — visionary programming, new way of using space | Public Thermal Transit Hub — civic infrastructure typology that does not yet exist. Spatial logic derived from UTCI gradient, not building conventions. |
| Architecture itself may not solve inequity | Thesis claims: demonstrating a building typology that operationalizes spatial equity under 2070 thermal conditions — not solving social inequality. The building is evidence, not a policy claim. |
| Energy problems may be solved by nuclear — don't anchor argument there | Argument is about spatial quality, thermal comfort, and public access — not energy scarcity. Thesis stands regardless of how energy is produced. |
| Too many 'what ifs' — commit to one | One position: projected climate data drives architecture. The hub is a programmatic consequence of that, not a separate hypothesis. |
| More factors to drive form — otherwise just typical passive design | Form drivers: UTCI floor shift + wet-bulb threshold hours + microburst wind loads + social thermal exposure mapping (district UHI × vulnerable population) — simultaneously, via MOO. Cannot be designed intuitively. |

# 6. Work Already Produced

- Pre-thesis research paper: theoretical lineage (Frazer → Schumacher), 7 computational method studies with case studies, workflow synthesis diagram with feedback loops.

- Thermal inequity research document: Bangkok UHI projections, district heat risk classification map, vulnerable population mapping (children + elderly by district), comparative city analysis (Jakarta, India, Singapore).

- UTCI experiment (unpresented): morphed EPW 2070 SSP5-8.5 via CURA-lab, Grasshopper UTCI analysis comparing present vs future on a Bangkok block. Key finding: future minimum exceeds current average — entire stress category shifted up.

- EPW field analysis table: all relevant fields (dry bulb, dew point, humidity, GHR, DNR, wind direction, wind speed) mapped to architectural implications for 2075–2100.

- Morphing method comparison: Simple Shift vs Bounded Weighted Stretch (BTWS) — understanding what each preserves and what changes, and that current results likely underestimate peak stress.

## Next Steps

- Present UTCI experiment to advisors as preliminary finding — frame as 'UTCI floor shift' discovery.

- Add second data layer: overlay district UHI risk map × vulnerable population density → site selection criteria.

- Commit to a specific district (high heat risk + high vulnerable population + transit accessibility).

- Define the spatial program in more detail — transition zone dimensions, thermal gradient sequence.

- Begin precedent study: Singapore hawker centres (thermal-civic hybrid), Japan cooling centers (policy-level refuge), kinetic facade projects (Al Bahr Towers).

# 7. Session Handoff Notes

Upload this document at the start of every new Claude session. Add notes below to brief Claude on what has changed since the last session.

| Date | Session Focus | Key Decisions / Updates |
| --- | --- | --- |
| June 2026 | Initial setup | Thesis brief created. UTCI experiment documented. Advisor feedback responses drafted. Typology: Public Thermal Transit Hub. |

Files produced so far (keep these in your thesis folder):

- 65020505_pre-thesis_computational_methods.pdf — pre-thesis research paper

- UHI_thermal_inequity.pdf — thermal inequity research document

- UTCI_experiment_2070.pdf — present vs future UTCI comparison (UNPRESENTED)

- thesis_brief.docx — this document

Atin Punyanitya  —  KMITL Architecture  —  Thesis Brief  —  June 2026