Fin & Shading Length Study

UTCI Performance vs. Vertical/Horizontal Shading Depth — 2070 SSP5-8.5 EPW

Atin Punyanitya  |  KMITL Architecture  |  Experiment Summary

# 1. Purpose

This experiment tests how the depth of combined vertical and horizontal shading fins affects outdoor thermal comfort (UTCI) on a generic Bangkok block, under the projected 2070 SSP5-8.5 climate scenario. The goal was to find the point at which extending the fins further stops producing a meaningful reduction in UTCI — the diminishing-returns threshold for passive shading.

# 2. Experiment Setup

Simulation run in Grasshopper with Ladybug UTCI analysis, using the morphed 2070 SSP5-8.5 EPW file as the climate input.

| Parameter | Value |
| --- | --- |
| Test date / time | 24 April, 14:00 (peak solar / heat-load condition) |
| Climate input | Morphed EPW, 2070, SSP5-8.5 |
| Analysis grid | 2.50 m grid size |
| Sensor height | 1.65 m (HumanToSky height) |
| Test points per iteration | 323 |
| Context geometry | Surrounding buildings, floor slabs, walls, tower mass, plus vertical and horizontal shading fins |
| Shading increment | Vertical and horizontal fins extended 20 cm per iteration, from 0.20 m up to 3.60 m |

Table 1. Simulation parameters.

# 3. Results

## 3.1 Per-Iteration UTCI Distribution

Across the 323 test points per iteration, the highest recorded UTCI value across all iterations was 46.28°C, occurring at the west and south-west grid points — consistent with low afternoon sun angles from those directions. The raw per-point plot shows a repeating ripple pattern, which is an artefact of how Grasshopper orders the test points (alternating south grid / north grid columns) rather than a real spatial signal.

Figure 1. UTCI per test point, all iterations overlaid. Peak 46.28°C at west/south-west.

## 3.2 Average Temperature vs. Shading Length

The site-wide average UTCI decreases as shading length increases, but the rate of improvement drops off sharply. Average temperature falls from about 41.96°C at 0.20 m of shading to about 41.41°C at 3.60 m — a total reduction of roughly 0.55°C across the entire 3.4 m range tested. Most of that gain occurs in the first 1.5–2.0 m; beyond that, each additional 20 cm buys only a fraction of a degree.

Figure 2. Average UTCI per iteration, plotted against shading length (m).

## 3.3 Min / Max Stabilisation

Tracking minimum, maximum, and mean UTCI per iteration revealed a stabilisation pattern:

- Minimum UTCI locks at 41.07064°C from 0.60 m of shading onward and does not change through 3.60 m.

- Maximum UTCI locks at 44.18714°C from 1.80 m through 3.40 m, then drops slightly at 3.60 m.

- Mean UTCI continues to decrease across the full range, but in progressively smaller increments.

Working logic applied: once an iteration produces no change (Δ = 0) across three consecutive data sets (min, max, mean), further shading extension at that depth was treated as not worth pursuing — since minimum values typically stop changing once shading extends past 1.00 m, and maximum reduction was prioritised because it represents the most extreme exposure point.

## 3.4 Selected Iterations

Based on the stabilisation pattern, three iterations were selected for detailed comparison: 1.40 m, 2.00 m, and 2.60 m of shading length.

Figure 3. Min / average / max UTCI for the three selected shading lengths.

| Temp. data | 1.40 m | 2.00 m | 2.60 m |
| --- | --- | --- | --- |
| Max. temp. (°C) | 44.64 | 44.27 (Δ 0.37) | 44.19 (Δ 0.45) |
| Min. temp. (°C) | 41.07 | 41.07 (Δ 0.00) | 41.07 (Δ 0.00) |
| Avg. temp. (°C) | 41.66 | 41.53 (Δ 0.13) | 41.50 (Δ 0.16) |

Table 2. Performance comparison of selected iterations, with Δ measured against the 1.40 m baseline.

Increasing shading length from 2.00 m to 2.60 m reduced peak temperature by a further 0.08°C and average temperature by a further 0.03°C — confirming the diminishing-returns pattern already visible in the average-temperature curve (Figure 2).

Figure 4. Full-day UTCI performance comparison: no shading vs. 1.40 m, 2.00 m, and 2.60 m shading.

# 4. Summary of Findings

- Shading is effective but bounded: extending fins from 0.20 m to 3.60 m reduces average UTCI by only ~0.55°C total, with most of that gain captured in the first 1.5–2.0 m.

- Minimum UTCI (41.07°C) is fixed from 0.60 m of shading onward — no amount of additional fin depth lowers it further at this sun angle and geometry.

- Maximum UTCI plateaus near 44.19–44.27°C from 1.80 m onward, with only marginal reduction (0.08°C) between 2.00 m and 2.60 m.

- The 1.40 m / 2.00 m / 2.60 m comparison set was selected as the range where additional shading depth produces measurable but rapidly diminishing returns.

- An open question remains whether the observed stabilisation in min/max values reflects a true geometric limit (the fin already blocks the maximum possible solar vector at this sun angle) or is partly an artefact of the 2.50 m analysis grid being too coarse to register smaller incremental shifts. This should be checked by re-running a subset of iterations at a finer grid resolution (e.g. 1.25 m) to see whether the locked values move.